CC 101 – Introduction to Computing

“Know Your Network Settings”: Exploring Windows 11 Network Configuration Tools

| Name: | Block: |
| --- | --- |
| Computer Unit Used (Desktop/Laptop, brand): | Date: |

## I.  Objectives

At the end of this activity, you should be able to:

- Locate network adapter information using Windows 11 Settings, the Control Panel, Command Prompt (CMD), and PowerShell.

- Use ipconfig, getmac, hostname, ping, netstat, route print, and systeminfo in CMD, and Get-NetAdapter, Get-NetIPAddress, and Test-Connection in PowerShell.

- Interpret a sample ipconfig /all output from a networked computer: identify its IP address, subnet mask, default gateway, and DNS servers, and classify the address as public or private.

- Relate each tool and finding to the layer of the OSI model where it works.

- Design a simple network and IP address plan for the computer laboratory.

## II.  Materials Needed

- A computer running Windows 11 (a network or Internet connection is not required for this activity)

- This worksheet (printed copy, or the digital file to fill out and submit)

- Week 6 lecture notes, for reference on IP addresses, protocols, the OSI model, and network devices

## III.  Background

A computer does not need to be connected to a network to have network settings. Every PC has one or more network adapters (NICs), each with a permanent MAC address, and Windows always runs the TCP/IP software, including the loopback address 127.0.0.1 (IPv4) and ::1 (IPv6) that the computer uses to talk to itself. Windows 11 lets you inspect these settings in four places: the Settings app, the classic Control Panel, Command Prompt (CMD), and PowerShell. In this activity, you will explore all four on your own unit, observe how Windows behaves when no network is available, and then analyze a sample configuration taken from a computer that is connected to a network.

## IV.  Procedure

Reminder: This is a viewing activity. Do not change, disable, or reset any network setting. Close every dialog with Cancel. Record values exactly as Windows shows them.

Part A – Windows 11 Settings

- Press Windows + I to open Settings, then click Network & internet. Write down the connection status shown at the top of the page (for example, “Not connected”).

- Click Advanced network settings. Under Network adapters, note every adapter listed (for example, Ethernet, Wi-Fi, Bluetooth Network Connection) and whether each one is enabled or disconnected.

- Scroll down and click Hardware and connection properties. For each adapter, record its Name, Description, Physical address (MAC), and Status in Table 1.

Part B – Control Panel (Network Connections)

- Press Windows + R, type ncpa.cpl, and press Enter. This opens the classic Network Connections window. A red X on an icon means the adapter has no network connection.

- Right-click your Ethernet adapter and choose Properties. List the items checked under “This connection uses the following items” in Table 2.

- Select Internet Protocol Version 4 (TCP/IPv4) and click Properties. Record whether the computer is set to Obtain an IP address automatically (DHCP) or Use the following IP address (static), and the same for DNS. Click Cancel, then Cancel again. Do not change anything.

Part C – Command Prompt (CMD)

- Press the Windows key, type cmd, and press Enter. Run each command below, one at a time, and record the results in Table 3.

hostname

getmac /v

ipconfig /all

Tip: For a disconnected adapter, ipconfig /all shows “Media disconnected” and no IPv4 address, but it still shows the Description and Physical Address.

- Test the computer’s own TCP/IP software using the loopback address. Record whether you got replies and the time. For ping localhost, note the address that replied.

ping 127.0.0.1

ping localhost

- Now observe what happens when there is no network. Record the exact message each command displays.

ping 8.8.8.8

nslookup google.com

- View the ports your computer is listening on and its routing table. In Table 3, record three LISTENING local addresses with their port numbers, and the Netmask shown for the Network Destination 127.0.0.0.

netstat -an | findstr LISTENING

route print -4

- Run the command below. It may take a few seconds. Scroll to Network Card(s) and record how many network cards are installed and the status of each.

systeminfo

Part D – PowerShell

- Right-click the Start button and choose Terminal (or press the Windows key, type PowerShell, and press Enter). Run each command below and record the results in Table 4.

Get-NetAdapter

Get-NetIPAddress -AddressFamily IPv4 | Format-Table InterfaceAlias, IPAddress, PrefixLength

Test-Connection 127.0.0.1 -Count 4

Get-NetTCPConnection -State Listen | Select-Object -First 5 LocalAddress, LocalPort

Tip: PowerShell commands follow a Verb-Noun pattern (Get-NetAdapter = “get the network adapters”). Compare each result with the matching CMD command from Part C.

Part E – Analyze a Networked Computer

- The output below was taken from a laboratory computer that is connected to a network. Study it, then complete Table 5.

Sample Output 1. ipconfig /all (excerpt) from a networked PC

| Host Name . . . : | CICT-LAB-PC07 |
| --- | --- |
| Ethernet adapter Ethernet: |  |
| Description . . . : | Realtek PCIe GbE Family Controller |
| Physical Address . . . : | 3C-52-82-A1-7F-09 |
| DHCP Enabled . . . : | Yes |
| Link-local IPv6 Address . . . : | fe80::8c2d:4a1f:93b2:61e5%12(Preferred) |
| IPv4 Address . . . : | 192.168.10.37(Preferred) |
| Subnet Mask . . . : | 255.255.255.0 |
| Lease Obtained . . . : | Wednesday, October 14, 2026 8:02:15 AM |
| Lease Expires . . . : | Thursday, October 15, 2026 8:02:15 AM |
| Default Gateway . . . : | 192.168.10.1 |
| DHCP Server . . . : | 192.168.10.1 |
| DNS Servers . . . : | 192.168.10.1, 8.8.8.8 |

Part F – Design a Network for Our Laboratory

- Our laboratory does not yet have a network. In the box after Table 5, draw a simple star topology network for it. Include at least four PCs, one switch, one router (default gateway), one printer, and the ISP / Internet. Use lines to show connections.

- Assign each device a private IPv4 address from the network 192.168.10.0/24, using 192.168.10.1 for the router. Label each device in your drawing and complete the IP Address Plan (Table 6).

Part G – Optional / Bonus

- In CMD, run type C:\Windows\System32\drivers\etc\hosts to view (not edit) the hosts file. In one or two sentences, explain what this file is for and how it relates to DNS.

## V.  Data and Findings

Table 1. Network Adapters in Settings (Part A)

Connection status shown on the Network & internet page: ______________________________

| Adapter Name | Description | Physical Address (MAC) | Status |
| --- | --- | --- | --- |

Table 2. Adapter Properties (Part B)

| Items Checked Under “This connection uses the following items” | Item Checked (continued) |
| --- | --- |

| TCP/IPv4 Setting | Automatic or Static? |
| --- | --- |
| IP address |  |
| DNS server address |  |

Table 3. Command Prompt Results (Part C)

| Command / Item | Your Findings | OSI Layer |
| --- | --- | --- |
| hostname |  |  |
| getmac /v: MAC address and Transport Name |  |  |
| ipconfig /all: Media State of Ethernet |  | — |
| ipconfig /all: DHCP Enabled |  | — |
| ping 127.0.0.1: replies and time |  |  |
| ping localhost: address that replied |  | — |
| ping 8.8.8.8: exact message |  |  |
| nslookup google.com: exact message |  |  |
| netstat -an: three LISTENING addresses:ports |  |  |
| route print -4: Netmask for 127.0.0.0 |  | — |
| systeminfo: number of NICs and status |  | — |

Table 4. PowerShell Results (Part D)

| Command | What It Showed | Matches CMD? (Y/N) |
| --- | --- | --- |
| Get-NetAdapter: Name, Status, MacAddress, LinkSpeed |  |  |
| Get-NetIPAddress: IPv4 addresses and PrefixLength |  |  |
| Test-Connection 127.0.0.1: result |  |  |
| Get-NetTCPConnection -State Listen: two LocalAddress:LocalPort pairs |  |  |

Table 5. Sample Output Analysis (Part E)

| Question | Your Answer |
| --- | --- |
| IPv4 address of PC07 |  |
| Subnet mask (and its /prefix) |  |
| Default gateway, and what device it usually is |  |
| DNS servers |  |
| Was the address assigned by DHCP or set manually? How do you know? |  |
| Public or private? Which RFC 1918 range? |  |
| IPv4 address of PC07 in binary (four octets) |  |
| MAC address, and the OSI layer where it is used |  |
| How long is the DHCP lease? |  |
| If the DHCP server failed, what kind of address would PC07 assign itself? |  |

Figure 1. Proposed Laboratory Network Diagram (Part F)

Draw or paste your diagram here.

Table 6. IP Address Plan (Part F)

| Device | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Router (gateway) | 192.168.10.1 | 255.255.255.0 | — |
| PC 1 |  |  |  |
| PC 2 |  |  |  |
| PC 3 |  |  |  |
| PC 4 |  |  |  |
| Printer |  |  |  |

Part G Notes (Optional / Bonus)

## VI.  Realization (Reflection)

In two to four sentences each, answer the following in your own words.

1. Even without a network connection, what network information did your computer still have? Why do you think it still has it?

2. Compare Settings, Control Panel, CMD, and PowerShell. Which tool did you find most useful for viewing network information, and why?

3. What did the messages from ping 8.8.8.8 and nslookup google.com tell you? How would you use messages like these when troubleshooting a real network?

4. Why should you be careful about sharing your MAC address, IP address, or ipconfig /all screenshots online? Relate your answer to data privacy.

## VII.  Scoring Guide

| Criteria | Description | Points |
| --- | --- | --- |
| Completeness | All tables (1–6) and required fields are filled out. | 25 |
| Accuracy | Recorded values match what the tools show and are consistent across Settings, CMD, and PowerShell; sample analysis, binary conversion, and OSI layers are correct. | 30 |
| Network Design | Diagram shows a star topology with PCs, switch, router, printer, and ISP; IP plan uses valid, non-duplicate addresses in 192.168.10.0/24. | 15 |
| Realization / Reflection | Answers are thoughtful, based on the student’s own results, and well written. | 20 |
| Neatness & Timeliness | Worksheet is legible/well-formatted and submitted on time. | 10 |
| Total |  | 100 |